NS II FRESHWATER PEARL MUSSEL SUB-BASIN MANAGEMENT PLANS

Biological Monitoring of Surface Water Quality in the Allow Catchment

December 2008

Conservation Services, Tullaha, Glenflesk, Killarney, Co. Kerry

Tel/Fax 064 6630130  e-mail cs@conservation-services.ie

CONTENTS

1	INTRODUCTION	3

2	METHODOLOGY	5

2.1	HABITAT ASSESSMENT	5

2.2	INVERTEBRATE SAMPLING AND WATER QUALITY ASSESSMENT			5

3	RESULTS	7

3.1	ALLOW MAIN CHANNEL	7

3.1.1	ALLOW SITE A	7

3.1.2	ALLOW SITE B	9

3.1.3	ALLOW SITE C	10

3.1.4	ALLOW SITE D	11

3.1.5	ALLOW SITE E	12

3.2	ALLOW TRIBUTARIES	13

3.2.1	ALLOW SITE 1	13

3.2.2	ALLOW SITE 2	14

3.2.3	ALLOW SITE 3	15

3.2.4	ALLOW SITE 4	16

3.2.5	ALLOW SITE 5	17

3.2.6	ALLOW SITE 6	18

3.2.7	ALLOW SITE 7	19

3.2.8	ALLOW SITE 8	20

3.2.9	ALLOW SITE 9	21

3.2.10	ALLOW SITE 10	22

3.2.11	ALLOW SITE 11	23

3.2.12	ALLOW SITE 12	24

3.2.13	ALLOW SITE 13	25

3.2.14	ALLOW SITE 14	26

3.2.15	ALLOW SITE 15	27

3.2.16	ALLOW SITE 16	28

3.2.17	ALLOW SITE 17	29

3.2.18	ALLOW SITE 18	30

3.2.19	ALLOW SITE 19	31

3.2.20	ALLOW SITE 20	32

3.2.21	ALLOW SITE 21	33

3.2.22	ALLOW SITE 22	34

3.2.23	ALLOW SITE 23	35

3.3	ALLOW RIVER MONITORING 1971-2008	36

4	CONCLUSIONS	38

5	REFERENCES	39

| APPENDIX 1 | HABITAT AT SAMPLING SITES |
| --- | --- |
| APPENDIX 2 | PHOTOGRAPHS |

# INTRODUCTION

Conservation Services, Freshwater Ecological Consultants have been commissioned by RPS Group to carry out biological sampling and water quality assessment in accordance with EPA Q-rating methodology at 28 biological water quality assessment sites in the Allow catchment in Co. Cork for the NS II freshwater pearl mussel sub-basin management plans project.

| ALLOW MAIN CHANNEL |  |
| --- | --- |
| Site code | Grid reference |
| AL A | 13131 11640 |
| AL B | 13653 11561 |
| AL C | 13930 11370 |
| AL D | 13948 10983 |
| AL E | 13815 10372 |
| ALLOW TRIBUTARIES |  |
| Site code | Grid reference |
| AL 1 | 13148 11776 |
| AL 2 | 13263 11605 |
| AL 3 | 13348 11612 |
| AL 4 | 13400 11609 |
| AL 5 | 13432 11557 |
| AL 6 | 13430 11572 |
| AL 7 | 13442 11551 |
| AL 8 | 13580 11542 |
| AL 9 | 13604 11558 |
| AL 10 | 13755 11580 |
| AL 11 | 13825 11555 |
| AL 12 | 13939 11403 |
| AL 13 | 13958 11311 |
| AL 14 | 13947 11302 |
| AL 15 | 13939 11229 |
| AL 16 | 13987 11119 |
| AL 17 | 13960 11121 |
| AL 18 | 13971 10944 |
| AL 19 | 13993 10838 |
| AL 20 | 13938 10693 |
| AL 21 | 13862 10637 |
| AL 22 | 13861 10571 |
| AL 23 | 13863 10512 |

Location of sampling sites is shown in Map 1.

# METHODOLOGY

## HABITAT ASSESSMENT

Habitat assessment was carried out at each of the sites selected for invertebrate/water quality assessment. These sites were assessed in terms of:

·	Stream width and depth

·	Substrate type, listing substrate fractions in order of dominance, i.e. large rocks, cobble, gravel, sand, mud etc.

·	Flow type, listing percentage of riffle, glide and pool in the sampling area

·	Instream vegetation, listing plant species occurring and their percentage coverage of the stream bottom at the sampling site

·	Dominant bankside vegetation, listing the main species overhanging the stream

·	Estimated summer cover by bankside vegetation, giving percentage shade of the sampling site

Grid references were recorded at all sites using a GPS. Site photographs were taken using a digital camera.

## INVERTEBRATE SAMPLING AND WATER QUALITY ASSESSMENT

A kick and stone wash invertebrate sample was taken at each site (ISO 7828:1985) using standard methodology employed by EPA. Each sample was retained in a large plastic bag at the sampling site. Sample processing and preservation was carried out under laboratory conditions within 24 hours of sampling. Mud was removed from each sample by sieving under running water through a 500μ sieve. Sieved samples were then live sorted for 30 minutes in a white plastic sorting tray under a bench lamp (ISO 5667-3:1994) and if necessary using a magnifying lens. Macroinvertebrates were stored in 70% alcohol. Preserved invertebrates were identified to the level required for the EPA Q-rating method (McGarrigle et al, 2002) using high-power and low-power binocular microscopes when necessary. The preserved samples were archived for future examination or verification. Based on the relative abundance of indicator species, a biotic index (Q-rating) was determined for each site in accordance with the biological assessment procedure used by the Environmental Protection Agency (Statutory Instruments No. 258 of 1998) and more detailed unpublished methodology (McGarrigle, Clabby and Lucey pers. comm.).

| Biotic Index | Water Quality | Quality Status |
| --- | --- | --- |
| Q5 | Good | Unpolluted Waters |
| Q4-5 | Fair - Good |  |
| Q4 | Fair |  |
| Q3-4 | Doubtful - Fair | Slightly Polluted Waters |
| Q3 | Doubtful | Moderately Polluted Waters |
| Q2-3 | Poor - Doubtful |  |
| Q2 | Poor | Seriously Polluted Waters |
| Q1-2 | Bad - Poor |  |
| Q1 | Bad |  |
| /0 | Suffix applied when there is evidence of an additional toxic influence on the invertebrate fauna |  |

# RESULTS

Q-ratings are estimated on the basis of the biological data and the nature of the habitat at the sampling site. Data on habitat at each sampling location are tabulated in Appendix 1 and photographs of each sampling site are presented in Appendix 2. Q-ratings for all sites are shown on Map 2.

## ALLOW MAIN CHANNEL

### ALLOW SITE A

| SITE CODE | AL A |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13131 11640 |
| Q-RATING | 4-5 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Chloroperlidae | 1 |
|  | Isoperla sp. | 1 |
|  | Ecdyonurus sp. | 3 |
|  | Rhithrogena sp. | 26 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 4 |
|  | Nemouridae | 2 |
|  | Goeridae | 23 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 22 |
|  | Baetis rhodani | 15 |
|  | Limnephilidae | 1 |
|  | Rhyacophilidae | 2 |
|  | Elmidae | 3 |
|  | Simuliidae | 3 |
|  | Tipulidae | 14 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | Tubificidae | 1 |
| Not assigned to an indicator group | Lumbricidae | 1 |

### ALLOW SITE B

| SITE CODE | AL B |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13653 11561 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 26 |
|  | Rhithrogena sp. | 10 |
| Group B - Moderately Pollution Sensitive | Taeniopterygidae | 3 |
|  | Leuctridae | 3 |
|  | Sericostomatidae | 2 |
| Group C - Moderately Pollution Tolerant | Ancylidae | 1 |
|  | Potamopyrgus antipodarum | 2 |
|  | Gammarus duebeni | 10 |
|  | Baetis rhodani | 58 |
|  | Hydropsychidae | 1 |
|  | Rhyacophilidae | 1 |
|  | Elmidae | 18 |
|  | Chironomidae (excl. Chironomus) | 6 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 9 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 2 |
|  | Ceratopogonidae | 2 |

### ALLOW SITE c

| SITE CODE | AL C |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 11930 11370 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 70 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 1 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 1 |
|  | Gammarus duebeni | 3 |
|  | Baetis rhodani | 119 |
|  | Glossosomatidae | 1 |
|  | Hydropsychidae | 10 |
|  | Polycentropodidae | 2 |
|  | Rhyacophilidae | 2 |
|  | Elmidae | 1 |
|  | Chironomidae (excl. Chironomus) | 5 |
|  | Simuliidae | 1 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 1 |
|  | Glossiphoniidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | None recorded |  |

### ALLOW SITE D

| SITE CODE | AL D |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13948 10983 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 57 |
|  | Heptagenia sp. | 1 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 4 |
| Group C - Moderately Pollution Tolerant | Ancylidae | 1 |
|  | Potamopyrgus antipodarum | 1 |
|  | Gammarus duebeni | 14 |
|  | Baetis rhodani | 8 |
|  | Hydropsychidae | 30 |
|  | Polycentropodidae | 7 |
|  | Rhyacophilidae | 2 |
|  | Dytiscidae | 1 |
|  | Elmidae | 3 |
|  | Chironomidae (excl. Chironomus) | 6 |
| Group D - Very Pollution Tolerant | Glossiphoniidae | 1 |
|  | Sphaeriidae | 2 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 18 |

### ALLOW SITE E

| SITE CODE | AL E |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13815 10372 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 21 |
| Group B - Moderately Pollution Sensitive | Baetidae (excl. Baetis rhodani) | 2 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 3 |
|  | Gammarus duebeni | 20 |
|  | Baetis rhodani | 60 |
|  | Hydropsychidae | 4 |
|  | Rhyacophilidae | 2 |
|  | Elmidae | 5 |
|  | Chironomidae (excl. Chironomus) | 13 |
|  | Simuliidae | 9 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 2 |

## ALLOW tributaries

### ALLOW SITE 1

| SITE CODE | AL 1 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13148 11776 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Chloroperlidae | 3 |
|  | Isoperla sp. | 3 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 18 |
|  | Nemouridae | 22 |
|  | Goeridae | 5 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 29 |
|  | Baetis rhodani | 1 |
|  | Limnephilidae | 2 |
|  | Rhyacophilidae | 9 |
|  | Elmidae | 1 |
|  | Gyrinidae | 1 |
|  | Chironomidae (ex. Chironomus) | 1 |
|  | Tipulidae | 5 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 6 |

### ALLOW SITE 2

| SITE CODE | AL 2 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13263 11605 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 2 |
|  | Rhithrogena sp. | 5 |
|  | Heptageniidae (small/damaged) | 1 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 10 |
|  | Goeridae | 4 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 38 |
|  | Baetis rhodani | 2 |
|  | Limnephilidae | 2 |
|  | Helodidae | 3 |
|  | Simuliidae | 10 |
|  | Tipulidae | 7 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | None recorded |  |

### ALLOW SITE 3

| SITE CODE | AL 3 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13348 11612 |
| Q-RATING | 4-5 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Chloroperlidae | 1 |
|  | Isoperla sp. | 1 |
|  | Ecdyonurus sp. | 4 |
|  | Rhithrogena sp. | 43 |
| Group B - Moderately Pollution Sensitive | Goeridae | 3 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 18 |
|  | Baetis rhodani | 15 |
|  | Limnephilidae | 1 |
|  | Rhyacophilidae | 4 |
|  | Elmidae | 10 |
|  | Chironomidae (excl. Chironomus) | 1 |
|  | Tipulidae | 3 |
| Group D - Very Pollution Tolerant | Glossiphoniidae | 2 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 1 |
|  | Lumbriculidae | 7 |

### ALLOW SITE 4

| SITE CODE | AL 4 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13400 11609 |
| Q-RATING | 3-4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | None recorded |  |
| Group B - Moderately Pollution Sensitive | Sericostomatidae | 5 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 61 |
|  | Baetis rhodani |  |
|  | Limnephilidae | 2 |
|  | Polycentropodidae | 1 |
|  | Elmidae |  |
|  | Simuliidae | 2 |
|  | Tipulidae | 3 |
| Group D - Very Pollution Tolerant | Glossiphoniidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Enchytraeidae | 1 |
|  | Lumbricidae | 2 |

### ALLOW SITE 5

| SITE CODE | AL 5 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13432 11557 |
| Q-RATING | 3-4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | None recorded |  |
| Group B - Moderately Pollution Sensitive | Leuctridae | 45 |
|  | Nemouridae | 2 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 11 |
|  | Baetis rhodani | 5 |
|  | Limnephilidae | 4 |
|  | Polycentropodidae | 2 |
|  | Rhyacophilidae | 2 |
|  | Helodidae | 2 |
|  | Chironomidae (excl. Chironomus) | 2 |
|  | Simuliidae | 40 |
|  | Tipulidae | 14 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | None recorded |  |

### ALLOW SITE 6

| SITE CODE | AL 6 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13430 11572 |
| Q-RATING | 2-3/0 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | None recorded |  |
| Group B - Moderately Pollution Sensitive | None recorded |  |
| Group C - Moderately Pollution Tolerant | Polycentropodidae | 2 |
|  | Chironomidae (excl. Chironomus) | 96 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 7 |
| Group E - Most Pollution Tolerant | Tubificidae | 10 |
| Not assigned to an indicator group | None recorded |  |

### ALLOW SITE 7

| SITE CODE | AL 7 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13442 11551 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 2 |
|  | Rhithrogena sp. | 9 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 45 |
|  | Nemouridae | 10 |
|  | Goeridae | 1 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 1 |
|  | Gammarus duebeni | 30 |
|  | Baetis rhodani | 13 |
|  | Limnephilidae | 2 |
|  | Philopotamidae | 8 |
|  | Polycentropodidae | 1 |
|  | Veliidae | 1 |
|  | Helodidae | 4 |
|  | Chironomidae (excl. Chironomus) | 1 |
|  | Simuliidae | 11 |
|  | Tipulidae | 5 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 2 |

### ALLOW SITE 8

| SITE CODE | AL 8 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13580 11542 |
| Q-RATING | 3-4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 1 |
|  | Rhithrogena sp. |  |
| Group B - Moderately Pollution Sensitive | Leuctridae | 5 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum |  |
|  | Gammarus duebeni | 51 |
|  | Baetis rhodani |  |
|  | Limnephilidae | 3 |
|  | Philopotamidae | 1 |
|  | Polycentropodidae | 2 |
|  | Veliidae | 1 |
|  | Helodidae | 2 |
|  | Chironomidae (excl. Chironomus) | 2 |
|  | Simuliidae |  |
|  | Tipulidae | 11 |
| Group D - Very Pollution Tolerant | Erpobdellidae |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Enchytraeidae | 5 |
|  | Lumbricidae | 1 |
|  | Ceratopogonidae | 1 |

### ALLOW SITE 9

| SITE CODE | AL 9 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13604 11558 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Isoperla sp. | 2 |
|  | Ecdyonurus sp. | 9 |
|  | Rhithrogena sp. | 6 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 1 |
|  | Nemouridae | 10 |
|  | Goeridae | 1 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 9 |
|  | Baetis rhodani | 38 |
|  | Ephemerellidae | 3 |
|  | Hydropsychidae | 4 |
|  | Polycentropodidae | 1 |
|  | Rhyacophilidae | 3 |
|  | Dytiscidae | 1 |
|  | Elmidae | 12 |
|  | Hydrophilidae | 1 |
|  | Chironomidae (excl. Chironomus) | 2 |
|  | Simuliidae | 2 |
|  | Tipulidae | 4 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 2 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 2 |

### ALLOW SITE 10

| SITE CODE | AL 10 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13755 11580 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 37 |
|  | Heptagenia sp. | 2 |
|  | Rhithrogena sp. | 2 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 1 |
|  | Goeridae | 1 |
|  | Sericostomatidae | 4 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 15 |
|  | Baetis rhodani | 5 |
|  | Hydropsychidae | 1 |
|  | Limnephilidae | 1 |
|  | Elmidae | 1 |
|  | Chironomidae (excl. Chironomus) | 6 |
|  | Simuliidae | 6 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 1 |
|  | Ceratopogonidae | 1 |

### ALLOW SITE 11

| SITE CODE | AL 11 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13825 11555 |
| Q-RATING | 3-4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Rhithrogena sp. | 6 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 15 |
|  | Goeridae | 8 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 25 |
|  | Baetis rhodani | 20 |
|  | Simuliidae | 11 |
|  | Tipulidae | 22 |
| Group D - Very Pollution Tolerant | Glossiphoniidae | 3 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 10 |

### ALLOW SITE 12

| SITE CODE | AL 12 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13939 11403 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 15 |
|  | Rhithrogena sp. | 1 |
| Group B - Moderately Pollution Sensitive | Baetidae (excl. Baetis rhodani) | 1 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 1 |
|  | Baetis rhodani | 104 |
|  | Ephemerellidae | 1 |
|  | Limnephilidae | 1 |
|  | Rhyacophilidae | 4 |
|  | Chironomidae (excl. Chironomus) | 16 |
|  | Simuliidae | 7 |
|  | Tipulidae | 2 |
| Group D - Very Pollution Tolerant | Glossiphoniidae | 3 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 6 |

### ALLOW SITE 13

| SITE CODE | AL 13 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13958 11311 |
| Q-RATING | 3-4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 5 |
|  | Rhithrogena sp. | 8 |
| Group B - Moderately Pollution Sensitive | Goeridae | 1 |
|  | Lepidostomatidae | 2 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 4 |
|  | Baetis rhodani | 12 |
|  | Elmidae | 1 |
|  | Chironomidae (excl. Chironomus) | 1 |
|  | Simuliidae | 1 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 6 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 5 |
|  | Naididae | 4 |

### ALLOW SITE 14

| SITE CODE | AL 14 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13947 11302 |
| Q-RATING | 4-5 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Isoperla sp. | 2 |
|  | Ecdyonurus sp. | 19 |
|  | Rhithrogena sp. | 9 |
| Group B - Moderately Pollution Sensitive | Goeridae | 1 |
|  | Odontoceridae | 1 |
|  | Sericostomatidae | 3 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 18 |
|  | Baetis rhodani | 2 |
|  | Ephemerellidae | 2 |
|  | Limnephilidae | 2 |
|  | Hydropsychidae | 18 |
|  | Polycentropodidae | 5 |
|  | Elmidae | 33 |
|  | Chironomidae (excl. Chironomus) | 1 |
|  | Simuliidae | 3 |
|  | Tipulidae | 11 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 1 |

### ALLOW SITE 15

| SITE CODE | AL 15 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13939 11229 |
| Q-RATING | 4-5 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Isoperla sp. | 2 |
|  | Ecdyonurus sp. | 35 |
|  | Rhithrogena sp. | 10 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 5 |
|  | Nemouridae | 30 |
|  | Lepidostomatidae | 2 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 1 |
|  | Gammarus duebeni | 13 |
|  | Hydracarina | 1 |
|  | Baetis rhodani | 4 |
|  | Limnephilidae | 2 |
|  | Hydropsychidae | 3 |
|  | Polycentropodidae | 1 |
|  | Rhyacophilidae | 4 |
|  | Elmidae | 1 |
|  | Chironomidae (excl. Chironomus) | 19 |
|  | Simuliidae | 15 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 2 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 1 |

### ALLOW SITE 16

| SITE CODE | AL 16 |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13987 11119 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 10 |
| Group B - Moderately Pollution Sensitive | Sericostomatidae | 3 |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 15 |
|  | Baetis rhodani | 21 |
|  | Limnephilidae | 1 |
|  | Polycentropodidae | 6 |
|  | Chironomidae (excl. Chironomus) | 1 |
|  | Tipulidae | 3 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 19 |

### ALLOW SITE 17

| SITE CODE | AL 17 |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13960 11121 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 4 |
|  | Rhithrogena sp. | 8 |
| Group B - Moderately Pollution Sensitive | Baetidae (excl. Baetis rhodani) | 1 |
|  | Sericostomatidae | 2 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 5 |
|  | Gammarus duebeni | 51 |
|  | Baetis rhodani | 10 |
|  | Glossosomatidae | 1 |
|  | Hydropsychidae | 6 |
|  | Philopotamidae | 3 |
|  | Polycentropodidae | 4 |
|  | Rhyacophilidae | 2 |
|  | Elmidae | 11 |
|  | Chironomidae (excl. Chironomus) | 4 |
|  | Simuliidae | 7 |
|  | Tipulidae | 3 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 3 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 1 |
|  | Lumbriculidae | 4 |

### ALLOW SITE 18

| SITE CODE | AL 18 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13971 10944 |
| Q-RATING | 3-4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Isoperla sp. | 1 |
|  | Ecdyonurus sp. | 5 |
|  | Rhithrogena sp. | 8 |
| Group B - Moderately Pollution Sensitive | None recorded |  |
| Group C - Moderately Pollution Tolerant | Ancylidae | 1 |
|  | Gammarus duebeni | 29 |
|  | Baetis rhodani | 48 |
|  | Rhyacophilidae | 3 |
|  | Elmidae | 6 |
|  | Simuliidae | 2 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 16 |
|  | Glossiphoniidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Ceratopogonidae | 1 |

### ALLOW SITE 19

| SITE CODE | AL 19 |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13993 10838 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 15 |
|  | Heptagenia sp. | 15 |
|  | Rhithrogena sp. | 18 |
| Group B - Moderately Pollution Sensitive | None recorded |  |
| Group C - Moderately Pollution Tolerant | Gammarus duebeni | 46 |
|  | Baetis rhodani | 101 |
|  | Hydropsychidae | 8 |
|  | Polycentropodidae | 1 |
|  | Rhyacophilidae | 4 |
|  | Dytiscidae | 1 |
|  | Elmidae | 34 |
|  | Chironomidae (excl. Chironomus) | 4 |
|  | Simuliidae | 10 |
|  | Tipulidae | 1 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbriculidae | 1 |

### ALLOW SITE 20

| SITE CODE | AL 20 |
| --- | --- |
| DATE OF SAMPLING | 4/10/08 |
| GRID REFERENCE | 13938 10693 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Isoperla sp. | 14 |
|  | Ecdyonurus sp. | 62 |
|  | Rhithrogena sp. | 27 |
| Group B - Moderately Pollution Sensitive | Leuctridae | 2 |
|  | Nemouridae | 1 |
|  | Goeridae | 6 |
|  | Sericostomatidae | 1 |
| Group C - Moderately Pollution Tolerant | Ancylidae | 1 |
|  | Gammarus duebeni | 22 |
|  | Baetis rhodani | 16 |
|  | Hydropsychidae | 1 |
|  | Limnephilidae | 1 |
|  | Polycentropodidae | 2 |
|  | Rhyacophilidae | 7 |
|  | Elmidae | 1 |
|  | Chironomidae (excl. Chironomus) | 4 |
|  | Simuliidae | 8 |
|  | Tipulidae | 5 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 13 |
|  | Glossiphoniidae | 1 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Nematomorpha | 1 |
|  | Lumbricidae | 1 |

### ALLOW SITE 21

| SITE CODE | AL 21 |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13862 10637 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Isoperla sp. | 2 |
|  | Ecdyonurus sp. | 48 |
|  | Rhithrogena sp. | 12 |
| Group B - Moderately Pollution Sensitive | Nemouridae | 1 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 1 |
|  | Baetis rhodani | 112 |
|  | Ephemerellidae | 1 |
|  | Hydropsychidae | 9 |
|  | Rhyacophilidae | 9 |
|  | Elmidae | 6 |
|  | Chironomidae (excl. Chironomus) | 20 |
|  | Simuliidae | 45 |
| Group D - Very Pollution Tolerant | None recorded |  |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 1 |
|  | Ceratopogonidae | 1 |

### ALLOW SITE 22

| SITE CODE | AL 22 |
| --- | --- |
| DATE OF SAMPLING | 10/10/08 |
| GRID REFERENCE | 13861 10571 |
| Q-RATING | 3 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | None recorded |  |
| Group B - Moderately Pollution Sensitive | Leuctridae | 2 |
| Group C - Moderately Pollution Tolerant | Potamopyrgus antipodarum | 211 |
|  | Gammarus duebeni | 110 |
|  | Polycentropodidae | 1 |
|  | Elmidae | 2 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 3 |
|  | Glossiphoniidae | 5 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 3 |
|  | Ceratopogonidae | 1 |

### ALLOW SITE 23

| SITE CODE | AL 23 |
| --- | --- |
| DATE OF SAMPLING | 20/11/08 |
| GRID REFERENCE | 13863 10512 |
| Q-RATING | 4 |

| INDICATOR GROUP | TAXON | Number |
| --- | --- | --- |
| Group A - Very Pollution Sensitive | Ecdyonurus sp. | 26 |
|  | Rhithrogena sp. | 28 |
|  | Heptageniidae (small/damaged) | 1 |
| Group B - Moderately Pollution Sensitive | Nemouridae | 2 |
|  | Sericostomatidae | 2 |
| Group C - Moderately Pollution Tolerant | Ancylidae | 2 |
|  | Gammarus duebeni | 31 |
|  | Hydracarina | 6 |
|  | Baetis rhodani | 14 |
|  | Hydropsychidae | 4 |
|  | Limnephilidae | 2 |
|  | Polycentropodidae | 1 |
|  | Rhyacophilidae | 3 |
|  | Elmidae | 1 |
|  | Helodidae | 1 |
|  | Chironomidae (excl. Chironomus) | 3 |
|  | Simuliidae | 8 |
|  | Tipulidae | 11 |
| Group D - Very Pollution Tolerant | Erpobdellidae | 5 |
| Group E - Most Pollution Tolerant | None recorded |  |
| Not assigned to an indicator group | Lumbricidae | 1 |

## ALLOW RIVER MONITORING 1971-2008

| EPA Site Code No. | CS Site Code | EPA DATA |  |  |  |  |  |  |  |  |  |  |  |  | CS DATA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1971 | 1974 | 1975 | 1978 | 1980 | 1982 | 1986 | 1990 | 1994 | 1997 | 2000 | 2003 | 2006 | 2008 |
| 18/A/02/0020 | A | - | - | - | - | - | - | - | 4-5 | 4-5 | 4-5 | 4-5 | 4-5 | 4 | 4-5 |
| - | 1 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 2 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 3 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4-5 |
| - | 4 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3-4 |
| - | 5 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3-4 |
| - | 6 | - | - | - | - | - | - | - | - | - | - | - | - | - | 2-3/0 |
| - | 7 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 8 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3-4 |
| - | 9 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| 18/A/02/0100 | B | 5 | - | 5 | 5 | 5 | - | 4-5 | 4-5 | 4-5 | 4-5 | 4-5 | 4-5 | 4-5 | 4 |
| - | 10 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 11 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3-4 |
| - | 12 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| 18/A/02/0200 | C | 3-4 | 3-4 | 4 | 3 | 4 | - | 4 | 3 | 4 | 4 | 4 | - | - | 4 |
| - | 13 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3-4 |
| - | 14 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4-5 |
| - | 15 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4-5 |
| - | 16 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 17 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| 18/A/02/0300 | D | 4-5 | 3 | 4 | 3-4 | 4 | 4 | 4 | 4 | 4 | 4-5 | 4-5 | 4-5 | 4-5 | 4 |
| - | 18 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3-4 |
| - | 19 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 20 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 21 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| - | 22 | - | - | - | - | - | - | - | - | - | - | - | - | - | 3 |
| - | 23 | - | - | - | - | - | - | - | - | - | - | - | - | - | 4 |
| 18/A/02/0400 | E | - | - | - | - | 3-4 | 3-4 | 3-4 | 5 | 4-5 | 4-5 | 4-5 | - | - | 4 |

EPA Data 1971 to 2003 from Clabby et al (2004). EPA data 2006 from J. Lucey (EPA) pers. comm.

# CONCLUSIONS

All sites assessed on the main channel of the Allow merited a Q-rating of Q4, with the exception of the most upstream site at Rowls Aldworth Bridge where a Q4-5 was recorded.

Of the twenty three tributaries assessed, twelve were Q4, six were Q3-4, three were Q4-5, one was Q3, and a single small watercourse was Q2-3/0.

Notably the largest tributary of the Allow, the Glashawee River, merited a Q-rating of Q4.

# REFERENCES

Clabby, K. et al (2004) Interim report on the biological survey of river quality. Results of the 2003 investigations. EPA.

McGarrigle, M. et al (2002)  Water Quality in Ireland 1998-2000. EPA.

APPENDIX 1

HABITAT ASSESSMENT AT SAMPLING SITES

APPENDIX 2

PHOTOGRAPHS